CodeUP  Software User Manual

- Extract the file. No installation is required. Open "CodeUP.exe" to run the software.

- Interface Overview

1>Firmware Selection Button

2>Firmware Information Display

3>Upgrade Button

4>Upgrade Status Information

5>Upgrade Result Display

6>Device Recovery Function

7>Upgrade Progress Bar

8>COM Port Selection

3.   Firmware Upgrade Procedure：

  - Power on the device and connect it to the computer using the programming cable.

  - Open Device Manager on the computer and confirm that the driver has been installed correctly.

  - Click the Firmware Selection button and select the firmware file to be updated.

  - Check the Firmware Information to confirm the firmware details.

  - Select the corresponding COM port of the programming cable at the bottom-left corner.

  - Click Upgrade to start the firmware update.If the upgrade is successful, the result will be displayed in green text: "Success".If the upgrade fails, the result will be displayed in red text: "Timeout".

4.    Notes

  - Do not disconnect the power supply or programming cable during the upgrade process.

  - If the handshake fails after clicking Upgrade, please check the connection of the programming cable and ensure the driver is installed correctly.

  - If the handshake is successful but the progress bar stops halfway and shows "Timeout", please check the power supply and programming cable connection. Then use the Fail Repair function to recover the device.